Lab 2

Secure LAN

ITSC 206: Advanced Networking for Offensive and Defensive Environments

Table of Contents

Lab Outcomes	4

Files/Download	4

Reading	4

Introduction	4

MAC Address Spoofing - Demo	4

ARP Address Spoofing (Poisoning)	5

VLAN Hopping	6

DHCP Starvation	7

DHCP Server Spoofing	9

STP Spoofing	10

Lab Outcomes

- Perform network setup and attack against network device.

- Learn how to use multiple tools to perform various network LAN attacks.

Lab Resources (Devices/OS/Files/Download)

- Windows 2012R2 VM – Either from instructor or locally on workstation

  - TFTPD64 software (rename from .txt to .exe) – D2L Student resources

  - TMAC – D2L Student resources

- Kali Linux with Yersinnia

- Cisco Switches (Cisco Catalyst 2960) within SAIT lab

Reading

- None

Introduction

This lab gives you practice in basic network attack focused on OSI layer 2 (MAC).

MAC Address Spoofing - Demo

Within Linux (Kali) use macchanger.

- Write down initial MAC on the Attacker PC (Kali):_______________________

- Execute command: macchanger -m 00:0c:29:1d:dc:58 eth0

Within Windows 10 use TMAC

- https://technitium.com/tmac/ - Windows MAC Changer

- Write down initial MAC on the Win10 PC (Test):_______________________

- Install and open TMAC tool

- Review http://blog.technitium.com/2011/05/tmac-issue-with-wireless-network.html

- Question: Why is changing the MAC address not always a good idea? _______________

ARP Address Spoofing (Poisoning)

Tools:

http://www.macfro.com/ethical-hacking-sniffing-attack/ (Cain&Able) Windows10 - demo

http://Cyberang3l.wikispaces.com/HOWTO+-+ARP+Poisoning+in+Linux (arpspoof & dsniff – Kali)

- View initial ARP cache on the Victim PC (Windows 2k12Rs)

- View initial ARP cache on the Attacker PC (Kali)

- View initial MAC Address-Table on the Cisco Catalyst 2950/60

Student1Sw1#show mac address-table dynamic

- Configure & start Wireshark to sniff on Attacker PC (Kali)

- Start arpspoof attack application on the Attacker PC (Kali)

arpspoof -t desktopIP serverIP

arpspoof -t serverIP desktopIP

- Start the MITM (ARP Poisoning) attack

- View initial ARP cache on the Victim PC (Windows 2k12Rs)

- View initial ARP cache on the Attacker PC (Kali)

- View initial MAC Address-Table on the Cisco Catalyst 2950/60

Student1Sw1#show mac address-table dynamic

- Question: List the MAC addresses:______________________

VLAN Hopping

Tools:

Yersinia - http://www.yersinia.net/index.net (Kali -DTP & DTP Trunk)

- View initial Trunk Table on the Cisco Catalyst 2950/60

Student1Sw1#show interfaces trunk

- Configure student1sw1 to have vlan 100 on switch port 24

student1sw1#config t

student1sw1(config)# interface fa0/1

student1sw1(config-if)# vlan 1

student1sw1(config-if)#end

student1sw1#config t

student1sw1(config)# interface fa0/24

student1sw1(config-if)# vlan 100

student1sw1(config-if)#end

- Use following command student1sw1# show interfaces trunk

Question: What is the displayed configuration? _________________

- Kali, enter from terminal window ping the IP address of the desktop or server, Question: Are you successful?  _______________

- Kali, enter from terminal window, Yersinia –G, click on Launch attack button on left, the choose attack pop up window, select enabling trunking, click OK

- Waiting three minutes should be enough time to capture DTP traffic (left pane)

- Kali, enter from terminal window, ping the IP address of the desktop or server, Question: Are you successful?  _______________

- Use following command student1sw1# show interfaces trunk

Question: What is the displayed configuration? _________________

- In Kali, from the Yersinia window, click on List attacks button on center, in the choose cancel all attacks from the Attack list pop up window

DHCP Starvation

http://itsecurity.telelink.com/dhcp-attacks/

Yersinia DHCP tools (Discovery)

- In the switch console window, run the following command: #show mac address-table dynamic

- Question: How many and what are the MAC addresses shown?__________________

- Configure Yersinia as pictured below by executing from Kali command shell: yersinia –G

- Then click on the DHCP tab in the window, then the Launch Attack button, which brings up the pop up as seen below

- Click on sending Discovery and then OK in the popup window,

- Leave run for twenty to thirty seconds, click on List Attacks, and cancel the DHCP Discovery Attack

- Within the switch console run: : #show mac address-table dynamic | begin criterion:

- Question: How many MAC addresses were added to the table?  ______________

- Question:  Will this method fill up the CAM table? __________

DHCP Server Spoofing

http://itsecurity.telelink.com/dhcp-attacks/

Yersinia DHCP tools (Discovery)

- In the switch console window, run the following command: #show mac address-table dynamic

- Question: How many and what are the MAC addresses shown?__________________

- It is expected that  yersinia –G has run and the Yersinia tool is available

- Then click on the DHCP tab in the window, then the Launch Attack button, which brings up the pop up as seen below

- Click on sending DHCP rogue server and then OK in the popup window,

- Another pop up window comes up with the DHCP details needed, click OK once

- Leave run for two to three minutes, click on List Attacks, and cancel the DHCP rogue server Attack

STP Spoofing

STP Network Diagram

http://www.ciscopress.com/articles/article.asp?p=1016582&seqNum=2

Attack 1: Taking Over the Root Bridge

Taking over a root bridge is probably one of the most disruptive attacks. The default STP bridge priority is 32768. Once in root attack mode, Yersinia sends a BPDU every 2 sec with the same priority as the current root bridge, but with a slightly numerically lower MAC address, which ensures it a victory in the root-bridge election process. Below shows Yersinia's STP attack screen, followed by a show command capture on the LAN switch under attack.

- Click on Claiming Root Role on Yersinia

- Perform the following commands on the switch:

Student1SW1#show spanning-tree summary totals

Student1SW1#show spanning-tree vlan 100 interface fa0/24

Student1SW1#show spanning-tree vlan 100 root

- Question: Does the RootId match the one on Yersinia: ______________

Securing The LAN

- On the switch enter the following commands:

Student1Sw1>enable

Student1Sw1# config terminal

Student1Sw1(config)# int range fa0/1-24

Student1Sw1(config -if-range)# switchport mode access

Student1Sw1(config -if-range)# switchport port-security maximum 3

Student1Sw1(config -if-range)# switchport port-security mac-address sticky

Student1Sw1(config -if-range)# switchport port-security violation restrict

Student1Sw1(config -if-range)#end

Student1Sw1#

- Perform the same Yersinia testing as above and gather the same evidence as above:

ARP Address Spoofing (Poisoning)

Add the following commands to the switch:

Student1Sw1(config)# ip arp inspection validate

A

- Question: List the MAC addresses:______________________

VLAN Hopping

On the switch enter the following commands:

Student1Sw1(config) )# int range fa0/1-24

Student1Sw1(config -if-range)#vlan 100

- Use following command student1sw1# show interfaces trunk

Question: What is the displayed configuration? _________________

- Kali, enter from terminal window ping the IP address of the desktop or server, Question: Are you successful?  _______________

- Kali, enter from terminal window, ping the IP address of the desktop or server, Question: Are you successful?  _______________

- Use following command student1sw1# show interfaces trunk

Question: What is the displayed configuration? _________________

DHCP Starvation

Add the following commands to the switch:

Switch(config)# ip dhcp snooping

Switch(config)# ip dhcp snooping vlan 100

- Question: How many and what are the MAC addresses shown?__________________

- Question: How many MAC addresses were added to the table?  ______________

- Question:  Will this method fill up the CAM table? __________

DHCP Server Spoofing

The extra protection should be implemented above

- Question: How many and what are the MAC addresses shown?__________________

STP Spoofing

On the switch enter the following commands:

Student1Sw1(config)# spanning-tree portfast bpduguard default

Student1Sw1(config-)#interface fa0/24

Student1Sw1(config-if)#spanning-tree portfast

Student1Sw1(config-if)#spanning-tree guard root

- Question: Does the RootId match the one on Yersinia: ______________

Blank page if necessary to make pages an even number

DO NOT DELETE THE SECTION BREAK BELOW. DELETING IT MAY IMPACT THE FORMATTING IN THIS DOCUMENT.