Lab 6

Firewall Lab- pfSense Setup

ITSC 206: Advanced Networking for Offensive and Defensive Environments

Table of Contents

Lab Setup	2

Files/Download	2

Reading	2

Introduction	2

Initial Firewall VM Setup	2

Firewall VM Setup	6

Extra Kali Setup	23

Lab Setup

- Review architecture for each configuration and optimize.

Files/Download

Reading

- https://doc.pfsense.org/index.php/Installing_pfSense#

Introduction

This document supports the setup of Lab6 firewall.

Initial Firewall VM Setup

Setup pfSense with the following attributes in the VM Player:

- New Virtual Machine

- Build from eval ISO (C:\Temp directory) (pfSense-CE-2.4.2-RELEASE-amd64.iso)

- Name of Virtual Machine: pfSense within VM into C:\Virtual Machine folder so everyone to use it

- Using 20 GB Hard disk

- Using 20 GB Hard disk

- Click on Customize Hardware button to change the VM

- Change to 1 GB of Ram, 1 CPU, 2x Network Adapter (LANSegment1, LANSegment2)

- After review of info, appears to be accurate, click on Finish

Firewall VM Setup

Setup pfSense with the following attributes in the VM Player:

- Start up VM server in VM Player, press 1 or let auto boot occur

- pfSense Installer Accept

- pfSense Install, press OK

- pfSense Installer, Keymap selection accept

- pfSense Installer disk partitioning, press OK

- pfSense Installer, Extracting distribution files

- pfSense Installer, Manual Configuration, press No

- pfSense Installer, Complete press Reboot

- pfSense, boot in multi user in 6 seconds

- pfSense setup, Option 2, set interface

- pfSense setup, available interfaces: 1, WAN IPv4 address: 172.16.4.1, subnet mask: 24

- pfSense setup,IPv6 DHCP: n, IPv6 address: <ENTER>

- Notice LAN & WAN interface and IP address

- Enter shell and review interfaces

- Once Web server is up and can ping the LAN interface of the pfSense Firewall, then use Firefox to http://192.168.1.1, login using default admin/pfsense

- pfSense Firewall setup, press Next

- pfSense Firewall setup, Netgate Gloabal support, step 1 of 9, press Next

- pfSense Firewall setup, General information, step 2 of 9,

- pfSense Firewall setup, Time server, step 3 of 9, press Next

- pfSense Firewall setup, Configure WAN interface

- pfSense Firewall setup, Configure WAN interface

- pfSense Firewall setup, Configure WAN interface , press Next

- pfSense Firewall setup, Configure LAN interface, press Next

- pfSense Firewall setup, Set Admin WebGUI Password, press Next

- pfSense Firewall setup, Reload configuration

- pfSense Firewall setup,

- pfSense Firewall Status Dashboard,

- pfSense Firewall, Firewall Rules

- pfSense Firewall NAT,

- a

Hints:

# Extra Kali Setup

- If it doesn’t already have set up a Kali workstation in a VM using ISO with the following parameters:

- 2 CPU

- 4 GB of RAM

- 80 GB HDD

  - Install an Kali by entering the following commands:

  - sudo apt install openvas

  - sudo openvas-setup

  - openvas-check-setup

DO NOT DELETE THE SECTION BREAK BELOW. DELETING IT MAY IMPACT THE FORMATTING IN THIS DOCUMENT.