Student Name:	Coleton Sanheim		Weight:	___2.0_____%

Student ID:	000862545		Marks: 	__________

Lab 6: Firewalls

# Learning Outcomes

- Configure zone-based policy firewalls.

- Perform firewall configuration and policy testing.

- Verify firewall functionality.

# Purpose

In this lab, we focus on the installation of a firewall and the configuration of access rules to allow traffic through the firewall.

# Tools

- VM Firewall (pfSense)

- VM workstation x 2

  - Kali-nmap-hping3-OpenVAS

  - Ubuntu-Apache

# References

https://doc.pfsense.org/index.php/Installing_pfSense#Prepare_Installation_Media

https://doc.pfsense.org/index.php/Category:PF

# Topology

VM Web

Server

pfSense

Firewall

Kali VM

LANseg2

LANseg1

© 2017, Southern Alberta Institute of Technology

| System | Network | IP Address |
| --- | --- | --- |
| PFSense | Trusted Net Untrusted Net | 192.168.1.1 172.16.4.1 |
| Kali | Untrusted | 172.16.4.98 |
| Ubuntu | Trusted | 192.168.1.2 |

# Setup

Before you begin the lab activities, perform the steps below to set up your computer.

- Set up a Kali workstation in a VM using ISO with the following parameters:

- 1 CPU

- 1 GB of RAM

- 80 GB HDD

- Network Adapter: NAT, for Internet access

- Latest version of Wireshark with Winpcap

- Reconfigure the network adapter:

- Change network adapter to LAN Segment 1 in the VM console

- Configure the IP address to: 172.16.4.98/24

- Configure the gateway to: 172.16.4.1

- Set up an Ubuntu (Web) server in the VM using ISO with the following parameters, use previous if available

- 1 CPU

- 1 GB of RAM

- 80 GB HDD

- Network Adapter: NAT, for Internet access

  - Install an Ubuntu web server by entering the following commands:

  - sudo apt install apache2

  - sudo apt install net-tools

- Reconfigure the network adapter:

- Change network adapter to LANSeg 2 in the VM console

You may need to perform sudo ifconfig <interfacename> down

- Configure the IP address to: 192.168.1.2/24

- Configure the gateway/dns to: 192.168.1.1

You may need to perform sudo ifconfig <interfacename> down

- Set up pfSense, see pfSense setup document:

- Set up LANSeg2 – MAC:0800275D9178 192.168.1.1/24 (LAN) em1

- Set up LANSeg1 – MAC:0800273560DB 172.16.4.1/24 (WAN) em0

# Activities

## Verify connectivity to gateways

Verify that the Web VM can reach gateway 192.168.1.1.

Verify the Kali VM can’t reach gateway 172.16.4.1.

## Verify that VM web server Apache2 is running

- From the VM web server console, telnet to localhost (127.0.0.1) on port 80.

Type GET /  HTTP/1.1 and press ENTER.

- You should receive HTML code from the web server.

## Reboot pfSense Firewall

## Configure pfSense to permit port 80 access from the VM Web Server

- Log into pfSense interface to configure firewall rules.

- default username admin and password pfsense to login

Under firewall > NAT > Port Forward, add a new rule to the OUTSIDE interface.

  - Permit WAN Interface

  - Protocol: TCP

  - Destination WAN address

  - Destination port range From: HTTP To: HTTP

  - Redirect target IP: 192.168.1.2

  - Redirect target port HTTP.

Change Firewall rules to ensure block is at the bottom

Click on Apply changes

## Verify VM workstation connectivity to the VM web server on port 80

- From the Kali VM workstation, open a browser to http://172.16.4.1.

- The Apache2 Ubuntu default page opens.

- Demonstrate connections to the web server using Wireshark.

- Question: What URIs are requested by the web browser?

- Hint: Wireshark, Apache log

## Ping the web server

- From VM workstation, ping the web server at 172.16.4.1.

- Your ping should be blocked by the pfSense firewall.

- Demonstrate that the ping was denied by the firewall using firewall logs, pfSense tcpdump

__________________________

- Demonstrate the ping sent by the VM web server using Wireshark.

__________________________

## Instructor verified

- Ask instructor to review setup and workings evidence: ___________________________

# Testing Activities

## Perform testing against the firewall from Kali

## Verify that the Web VM can reach gateway 192.168.1.1.

Suggested commands

nmap

-sS TCP Syn Scan

-sU UDP Scan

-p1-65535 ports 1-65535

-Pn Treat all hosts as online -- skip host discovery

-n Never do DNS resolution/Always resolve [default: sometimes]

-sV Probe open ports to determine service/version info

-T 3 Set timing template (higher is faster)

-v Increase verbosity level (use -vv or more for greater effect)

-A Enable OS detection, version detection, script scanning, and traceroute

-f; --mtu <val>: fragment packets (optionally w/given MTU)

-g/--source-port <portnum>: Use given port number

-oA <filename> Output in the three major formats at once

nmap -sS -p1-65535 -Pn -n -sV -T 3 -v -A 172.16.4.1 -oA nmap-S-pAll-172.16.4.1-2018-03-22

nmap -sU -p1-65535 -Pn -n -sV -T 3 -v -A 172.16.4.1 -oA nmap-U-pAll-172.16.4.1-2018-03-22

So I ran this one for ~3 hours and it got 1.01% done, and with an estimated time of 90 hours I decided to not finish it, sorry.

nmap -sS -g53 -p80 -f -Pn -n -sV -T 3 -v -A 172.16.4.1 -oA nmap-S-pAll-frag-172.16.4.1-2018-03-22

hping3

hping3 [destination host] [port] [number of packets to transmit] [verbose] [-S for SYN]

hping3 172.16.4.1 -p 22 -c 4 -V –S

## Gather Evidence Using

Screenshots or files of above output:_________________________

Using Firewall Web GUI - Status> Traffic Graph

Using Firewall Command line interface - [admin@pfSense.localdomain]/var/log: clog -f filter.log

## Vulnerability Assessment

# Activities

- Verify connectivity to gateways

Verify that the Kali VM can reach gateway 172.16.4.1

If browser didn’t pop up after install open with OpenVAS login screen, goto https://127.0.0.1:9392/

  - Login with creds from the setup file possible 32 character

  - Use OpenVAS browser goto Scans, use 172.16.4.1, output results